# 🚪 Smart IoT RFID & Keypad Door Lock System

An advanced, internet-connected security solution I engineered using the **ESP8266 (NodeMCU)** platform. This project bridges physical security with cloud-based control, featuring multi-factor authentication (MFA), local flash data persistence, and remote maintenance via OTA.

---

## 📑 Table of Contents

1. [Project Gallery & Visuals](#-project-gallery--visuals)

2. [System Logic Flow](#-system-logic-flow)

3. [Key Features](#-key-features)

4. [Hardware Architecture](#-hardware-architecture)

5. [Engineering Trade-offs & Component Selection](#-engineering-trade-offs--component-selection)

6. [Technical Challenges & Solutions](#-technical-challenges--solutions)

7. [Installation & Setup](#-installation--setup)

8. [Future Scope](#-future-scope)

9. [Full Source Code](#-full-source-code)

---

### 🎥 Project Demo Video

Click the image below to watch the full system in action:

[![Smart Lock Demo Video](./images/video_thumbnail.jpg)](https://www.youtube.com/watch?v=YOUR_VIDEO_ID)

## 🛠️ Project Gallery & Visuals

I have documented the physical construction and digital interface to provide a transparent view of the system’s integration.

### I. Hardware Implementation

![Physical Build View](./images/connections.jpg)

>Shows the Breadboard View.

### II. Circuit Schematic

I mapped the hardware to avoid GPIO pins that interfere with the ESP8266 boot process.

![Circuit Schematic](./images/crkt.png)

> **Circuit Diagram:** Clean schematic detailing the connections between the NodeMCU, MFRC522, IR sensor, and Active-Low Relay.

### III. Digital Interface (Blynk IoT)

I designed a custom mobile dashboard to manage the lock remotely and view real-time logs.

![Blynk App Layout](./images/blynk_dashboard.png)

> **Blynk Dashboard:** Features a Remote Unlock switch, Live Event Terminal, and Storage Management controls.

---

## 📂 System Logic Flow

To ensure the system is both secure and responsive, I designed a specific logic flow that prioritizes power efficiency and strict security validation.

LOGIC DESIGN:-

---[Standby Mode] -->|IR Sensor Detection| -->[Wake-up: Active Mode 10s]

-->{Authentication}

-->|RFID Scan| ->[Compare UID] / |Blynk Keypad| ->[Compare Master Code]

-->|Match| [GRANTED: Unlock Solenoid]

-->|Mismatch| [DENIED: Sound Buzzer]

-->[Log Event to LittleFS + NTP Sync]

-->[Log Denial + Increment Attempts]

-->|Attempts >= 3| -->[Intruder Alert + Lockdown]

-->[Standby Mode]

---

## 🔑 Key Features

* **Dual-Layer Authentication:** Physical access via **RFID (MFRC522)** tags combined with a **Virtual Keypad** on the Blynk App.

* **Remote Management:** Real-time lock/unlock capabilities and system status monitoring via the **Blynk IoT Cloud**.

* **Intelligent Power Management:** IR proximity sensor triggers "Active" mode, saving processor cycles during standby.

* **Event Logging:** Persistent log history stored locally in flash memory (**LittleFS**) with **NTP-synchronized** timestamps.

* **Over-the-Air (OTA) Updates:** Wireless firmware updates to eliminate the need for physical access to the module after installation.

* **Intruder Alert:** Automated system lockdown and cloud notifications triggered after 3 consecutive failed attempts.

---

## 🔌 Hardware Architecture

| Component | NodeMCU (ESP8266) Pin | Role |

| :--- | :--- | :--- |

| **Relay (Solenoid)** | **D3** | Actuator control (Active-Low) |

| **Buzzer** | **D4** | Audible feedback |

| **Emergency Button** | **RX (GPIO 3)** | Physical override (Safe for RX during boot) |

| **IR Sensor** | **D1** | System Wake-up/Proximity detection |

| **RFID Reader (SDA/SS)** | **D8** | SPI Chip Select |

| **RFID Reader (SCK)** | **D5** | SPI Clock |

| **RFID Reader (MOSI)** | **D7** | SPI Master Out Slave In |

| **RFID Reader (MISO)** | **D6** | SPI Master In Slave Out |

| **RFID Reader (RST)** | **D2** | SPI Reset |

| **LED** | **D0** | Visual Status Indication |

---

## ⚙️ Engineering Trade-offs & Component Selection

I specifically chose the **ESP8266 (NodeMCU)** over the standard Arduino Uno for several technical reasons:

* **Integrated Connectivity:** Built-in WiFi stack allowed for NTP time-stamping and cloud synchronization without requiring external shields.

* **Memory Management:** 4MB of onboard Flash allowed for **LittleFS** implementation, ensuring local data persistence during power outages.

* **Efficiency:** High clock speed (80MHz) ensured low-latency processing of SPI data while simultaneously handling WiFi background tasks.

---

## ⚠️ Technical Challenges & Solutions

### 1. Resolving Pin Conflicts (Internal Flash)

* **Problem:** Initially, the buzzer was connected to SD3, which caused continuous boot loops.

* **Solution:** I diagnosed the conflict (SD3 is tied to the internal SPI flash) and relocated the buzzer to **D4** and the button to **RX** to ensure stable boot states.

### 2. Serial Interface Conflicts

* **Problem:** Using the default RX/TX pins for output devices prevented code uploads via USB.

* **Solution:** I moved the buzzer to D4 and utilized the button on RX. Because a physical push-button acts as an "open circuit" by default, it no longer interferes with the USB-to-Serial data transfer.

### 3. Implementing Wireless Maintenance (OTA)

* **Problem:** Physical access to the USB port is difficult post-installation.

* **Solution:** I integrated **ArduinoOTA**, allowing me to push firmware updates wirelessly over the local network.

---

## 🚀 Installation & Setup

To replicate this firmware, the following C++ libraries are required via the Arduino Library Manager:

* `Blynk` by Volodymyr Shymanskyy

* `MFRC522` by GithubCommunity

* `NTPClient` by Fabrice Weinberg

### Blynk Configuration

1. Create a new template in the Blynk Web Dashboard.

2. Set up the following Datastreams (Virtual Pins):

* `V12` - Terminal (String)

* `V13` - Switch (Integer 0/1) for Remote Lock/Unlock

* `V14` - Switch (Integer 0/1) for Manual Buzzer

* `V16` - Button (Integer 0/1) for View Logs

* `V17` - Button (Integer 0/1) for Clear Logs

* `V0 to V11` - Buttons for Virtual Keypad (0-9, Clear, OK)

---

## 🔮 Future Scope

* **Migration to ESP32:** Transitioning the architecture to utilize integrated BLE (Bluetooth Low Energy) and dual-core processing.

* **Biometric Integration:** Adding a capacitive fingerprint sensor for a third factor of authentication in high-security environments.

* **Camera Integration:** Utilizing an ESP32-CAM module to capture and log photos of individuals scanning tags.

---

## 💻 Full Source Code

```cpp

#define BLYNK_TEMPLATE_ID "TMPL3nsQ49PWe"

#define BLYNK_TEMPLATE_NAME "Smart Lock"

#define BLYNK_AUTH_TOKEN "sdKbLaxBHUlhmgDcjdUte-ZJne3oiJ9X"

#include <ESP8266WiFi.h>

#include <BlynkSimpleEsp8266.h>

#include <MFRC522.h>

#include <SPI.h>

#include <LittleFS.h>

#include <NTPClient.h>

#include <WiFiUdp.h>

#include <ArduinoOTA.h>

char ssid[] = "ASIF CHAUDHRY";

char pass[] = "11223344";

#define PIN_IR        D1

#define PIN_RELAY     D3

#define PIN_BUZZER    D4

#define PIN_LED_RED   D0

#define PIN_EMERGENCY 3

#define SS_PIN        D8

#define RST_PIN       D2

MFRC522 mfrc522(SS_PIN, RST_PIN);

WiFiUDP ntpUDP;

NTPClient timeClient(ntpUDP, "pool.ntp.org", 19800);

String masterCode = "1234";

const String cardUID = "D9 D0 C8 06";

String enteredCode = "";

int attempts = 0;

bool systemActive = false;

bool firstConnect = true;

unsigned long activationTimer = 0;

const long activePeriod = 10000;

void ledSignal(String type) {

if (type == "GRANTED") {

for (int i = 0; i < 10; i++) { digitalWrite(PIN_LED_RED, !digitalRead(PIN_LED_RED)); delay(200); }

} else if (type == "DENIED") {

digitalWrite(PIN_LED_RED, HIGH); delay(2000);

} else if (type == "ALERT") {

for (int i = 0; i < 10; i++) { digitalWrite(PIN_LED_RED, HIGH); delay(200); digitalWrite(PIN_LED_RED, LOW); delay(200); }

}

digitalWrite(PIN_LED_RED, LOW);

}

void logToInternal(String message) {

timeClient.update();

String timestamp = timeClient.getFormattedTime();

File logFile = LittleFS.open("/log.txt", "a");

if (logFile) {

logFile.println("[" + timestamp + "] " + message);

logFile.close();

}

}

void handleSuccess(String method) {

attempts = 0; systemActive = false;

Blynk.virtualWrite(V12, "GRANTED: " + method);

logToInternal("GRANTED: " + method);

digitalWrite(PIN_RELAY, LOW);

Blynk.virtualWrite(V13, 1);

digitalWrite(PIN_BUZZER, HIGH); delay(200); digitalWrite(PIN_BUZZER, LOW);

ledSignal("GRANTED");

delay(5000);

digitalWrite(PIN_RELAY, HIGH);

Blynk.virtualWrite(V13, 0);

}

void handleFailure(String reason) {

attempts++;

String failMsg = "Denied: " + reason;

Blynk.virtualWrite(V12, failMsg);

logToInternal(failMsg);

digitalWrite(PIN_BUZZER, HIGH);

if (attempts >= 3) {

Blynk.logEvent("intruder_alert");

Blynk.virtualWrite(V12, "SYSTEM LOCKDOWN");

ledSignal("ALERT");

attempts = 0;

} else {

ledSignal("DENIED");

}

digitalWrite(PIN_BUZZER, LOW);

}

BLYNK_CONNECTED() {

if (firstConnect) {

Blynk.virtualWrite(V12, "ACTIVE");

digitalWrite(PIN_BUZZER, HIGH); delay(100); digitalWrite(PIN_BUZZER, LOW);

delay(100);

digitalWrite(PIN_BUZZER, HIGH); delay(100); digitalWrite(PIN_BUZZER, LOW);

firstConnect = false;

}

timeClient.update();

}

BLYNK_WRITE(V13) {

bool s = param.asInt();

digitalWrite(PIN_BUZZER, HIGH); delay(150); digitalWrite(PIN_BUZZER, LOW);

digitalWrite(PIN_RELAY, s ? LOW : HIGH);

if(s) {

Blynk.virtualWrite(V12, "Remote Unlock");

logToInternal("APP UNLOCK");

ledSignal("GRANTED");

} else {

Blynk.virtualWrite(V12, "Remote Lock");

logToInternal("APP LOCK");

}

}

BLYNK_WRITE(V14) {

int buzzerState = param.asInt();

if (buzzerState == 1) {

digitalWrite(PIN_BUZZER, HIGH);

Blynk.virtualWrite(V12, "Buzzer ON");

} else {

digitalWrite(PIN_BUZZER, LOW);

Blynk.virtualWrite(V12, "Buzzer OFF");

}

}

BLYNK_WRITE(V16) {

if (param.asInt() == 1) {

File logFile = LittleFS.open("/log.txt", "r");

if (!logFile) { Blynk.virtualWrite(V12, "No logs."); return; }

Blynk.virtualWrite(V12, "--- LOG HISTORY ---");

while (logFile.available()) { Blynk.virtualWrite(V12, logFile.readStringUntil('\n')); }

logFile.close();

}

}

BLYNK_WRITE(V17) {

if (param.asInt() == 1) {

LittleFS.remove("/log.txt");

Blynk.virtualWrite(V12, "Storage Cleared");

}

}

BLYNK_WRITE_DEFAULT() {

int pin = request.pin;

if (param.asInt() == 1 && systemActive && pin <= 11) {

if (pin <= 9) {

enteredCode += String(pin);

digitalWrite(PIN_BUZZER, HIGH); delay(80); digitalWrite(PIN_BUZZER, LOW);

}

else if (pin == 10) enteredCode = "";

else if (pin == 11) {

if (enteredCode == masterCode) handleSuccess("Keypad");

else handleFailure("Wrong Code");

enteredCode = "";

}

}

}

void setup() {

pinMode(PIN_RELAY, OUTPUT);

pinMode(PIN_BUZZER, OUTPUT);

pinMode(PIN_IR, INPUT);

pinMode(PIN_LED_RED, OUTPUT);

pinMode(PIN_EMERGENCY, INPUT_PULLUP);

digitalWrite(PIN_RELAY, HIGH);

digitalWrite(PIN_BUZZER, LOW);

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

ArduinoOTA.setHostname("SmartDoorLock-Production");

ArduinoOTA.begin();

SPI.begin();

mfrc522.PCD_Init();

LittleFS.begin();

timeClient.begin();

}

void loop() {

Blynk.run();

ArduinoOTA.handle();

if (digitalRead(PIN_EMERGENCY) == LOW) {

delay(50);

if (digitalRead(PIN_EMERGENCY) == LOW) {

bool isLocked = digitalRead(PIN_RELAY);

digitalWrite(PIN_RELAY, !isLocked);

Blynk.virtualWrite(V13, isLocked);

digitalWrite(PIN_BUZZER, HIGH); delay(100); digitalWrite(PIN_BUZZER, LOW);

logToInternal(isLocked ? "BTN UNLOCK" : "BTN LOCK");

while(digitalRead(PIN_EMERGENCY) == LOW) delay(10);

}

}

if (digitalRead(PIN_IR) == LOW && !systemActive) {

systemActive = true;

activationTimer = millis();

digitalWrite(PIN_BUZZER, HIGH); delay(20); digitalWrite(PIN_BUZZER, LOW);

Blynk.virtualWrite(V12, "System Active");

}

if (systemActive && (millis() - activationTimer > activePeriod)) {

systemActive = false;

Blynk.virtualWrite(V12, "System Standby");

}

if (systemActive && mfrc522.PICC_IsNewCardPresent() && mfrc522.PICC_ReadCardSerial()) {

String uid = "";

for (byte i = 0; i < mfrc522.uid.size; i++) {

uid += String(mfrc522.uid.uidByte[i] < 0x10 ? "0" : "");

uid += String(mfrc522.uid.uidByte[i], HEX);

if (i < mfrc522.uid.size - 1) uid += " ";

}

uid.toUpperCase();

if (uid == cardUID) handleSuccess("RFID");

else handleFailure("Wrong Card (" + uid + ")");

mfrc522.PICC_HaltA();

}

}